WP4.1 work done by CNRS

on Robotic Submerged Microhandling Controlled by pH switching

presented in St Louis, MO, USA.

Intelligent Robot and Systems Conference: IROS 2009

The 2009 IEEE/RSJ International Conference on Intelligent RObots and Systems was held at St Louis, Missouri USA from October 11 to 15, 2009. The conference theme is ‘Exploring New Horizons in Intelligent Robots and Systems’ reflecting the growing spectrum and recent developments in intelligent robots and systems (www.iros09.mtu.edu). More than 900 scientific papers were presented during this event which represents one of the best conferences in Robotics.

Interest for Hydromel

As the Hydromel project focuses on hybrid methods between self-assembly and robotic assembly, results of the project must be presented in the robotic community. The IROS conference is one of the biggest conferences in robotics and thus represents a good opportunity to present scientific results done in Hydromel. The presentation focuses on the current activities done in the workpackage WP4.1 of Hydromel. The presentation entitled ‘Robotic Submerged Microhandling Controlled by pH swithching’ and proposed by Jérôme Dejeu, Patrick Rougeot, Michaël Gauthier, Wilfrid Boireau (CNRS partner) was done by Michaël Gauthier on Monday 12th.

Presentation abstract

Principe of the microhandling controlled by pH switching

Robotic microhandling is a promising way to assemble microcomponents in order to manufacture new generation of Hybrid Micro ElectroMechanical Systems (HMEMS). However, in the scale of several micrometers, adhesion phenomenon highly perturbs the release and the positioning of the micro-object. This phenomenon is directly linked to the chemical composition of the surface of both the object and the gripper. We propose to control adhesion by using chemical self-assembly monolayer (SAM) on both surfaces. Different types of chemical functionnalisation is presented in this paper: a polyelectrolyte: Poly(Allylamine Hydrochloride) (PAH), and some silanes: 3 (ethoxydimethylsilyl) propyl amine (APTES) and (3 aminopropyl) triethoxysilane (APDMES). Adhesion has been compared between dry medium and wet medium and we show that the pH of the liquid can be used to modify the adhesion and to switch from an attractive behavior to a repulsive behavior.

The control of the pH can thus be used to increase adhesion during handling and cancel adhesion during release. This work shows the relevance of a new type of reliable robotic microhandling controlled by chemical properties of a liquid.