Lab automation 2009 January 24-28 2009 California

Poster presentation

| Code | MP32 |
| --- | --- |
| Track | High-Throughput Technologies |
| Title | Fully Automated Approach of a High Throughput Microinjection System |
| Author | Mr. Siegfried Graf, M.S. |
| Co-Authors | Siegfried F. Graf, Centre Suisse d'Electronique et de Microtechnique, Alpnach, Switzerland Helmut F. Knapp, Centre Suisse d'Electronique et de Microtechnique, Alpnach, Switzerland Ruoya Li, Bioprédic International, Rennes, France |
| Abstract | An innovative system to meet the need of drug researchers is developed in this project. The novel approach combines novel concepts from microrobotics and microfluidics to achieve a high throughput (HT) rate which will replace time consuming and costly manual operations used today. The complete cycle of manipulation for individual cells in suspension from sorting/singularization over immobilization, microinjection and releasing up to quality control are being regarded. To achieve a HT rate, established microfluidics systems for preparation and handling of cell suspensions are combined with microrobotic systems specialized to address, handle and observe individual cells in solution. The first fully automated system focuses on large cells like Xenopus oocytes. A specially designed flow cytometer for large opaque cells is combined with a carousel based microinjection system. The viability of cells is quality-controlled by a vision-algorithm within the flow cytometer. Viable cells are then transferred on demand to the microinjection system where individual cells are reversible immobilized and injected guided by a vision-feedback system. The fully automated system allows reducing the microinjection cycle from 300 seconds (manual approach) down to 6 seconds or less (automated approach) per cell. Further development steps are the integration of a force-feedback system for the microinjection to further enhance the success rate and the adaptation for injecting hard-to-transfect-cells. |